# Design and Implementation of a Non-coupled Integrated Transformer for a Two-phase Interleaved Flyback PFC Rectifier for USB PD 3.1 Applications

Karl Raymond Dela Cruz Roque, Student Member, IEEE, Haruki Hirasawa, Sihoon Choi, Member, IEEE, Yu Yonezawa, Member, IEEE, Jun Imaoka, Member, IEEE and Masayoshi Yamamoto, Member, IEEE

Abstract—Universal serial bus power delivery (USB PD 3.1) is a widely-applied standard for a lot of electronic device charging. In 2021, USB PD 3.1 was announced with a variable output of 5 V to 48 V and a maximum output power of 240 W. A multi-phase flyback rectifier is capable of power factor correction, galvanic isolation, and wide output voltage range, and is proposed as an alternative topology for USB PD. However, the number of magnetic components increases with the number of phases, which can lead to lower power density. This paper proposes the design of a non-coupled integrated transformer for a two-phase interleaved flyback rectifier to reduce the size and number of magnetic components. The independent operation of the integrated transformer in two phases was verified using a 240-W two-phase interleaved flyback PFC rectifier at 100 kHz switching frequency operating at discontinuous conduction mode, both in simulation and experiment. After conducting a volume analysis through the area product method, the proposed transformer is up to 19% smaller compared to its conventional counterpart.

Index Terms—Integrated magnetics, Transformer design, two-phase interleaved flyback rectifier, USB PD 3.1, PFC, Core volume analysis

# I. INTRODUCTION

U

NIVERSAL serial bus power delivery (USB PD) is a technology used for convenient, portable, fast charging in electronic devices. The latest standard, USB PD 3.1, was announced in 2021, with the maximum power supplied being up to 240 W, allowing for higher power applications [1]. Adapters for USB PD 3.1 must be capable of power factor correction (PFC) for supporting 75 W and above, be equipped with galvanic isolation for user safety, and be capable of providing an output voltage of 5 V to 48 V [2].

A flyback rectifier has been widely used for commercial adapters due to its simple configuration, easy PFC control, and wide output voltage range. Improved configurations of flyback rectifiers such as active clamp flyback converters [6]-[7] and bridgeless flyback rectifiers have also been proposed for higher converter efficiency [8]-[11]. However, they are generally more practical for lower power applications due to the inherent high current stress on the switches by these topologies.

Several power supply configurations for USB PD 3.1 have been proposed to meet the requirements. One example is a combination of a PFC rectifier, an LLC converter, and a buck converter as shown in Fig. 1(a) [3]-[4]. The LLC converter is not suitable for wide output voltage and requires a buck converter in its subsequent stage, though it realizes high power density. Three stages of power conversion also lead to many circuit components and therefore high cost. An asymmetrical half bridge (AHB) flyback converter in the second stage of a boost PFC circuit was introduced as shown in Fig. 1(b) [5]. The AHB allows for zero-voltage switching (ZVS) by the application of resonant capacitors in series placed on the primary side of the transformer. However, the design of this circuit is complicated especially that the resonant design must take into account the leakage inductance of the transformer.

Interleaving rectifiers have also been considered to increase the maximum output power. Two-phase interleaving allows half of the total power to be supplied in each phase, therefore reducing the possible stress on the circuit elements [12]. The topology is shown in Fig. 1 (c), where the two phases are parallel in the input and output side. The source nodes of switches S1 and S2 are common, only one power supply is required for gate drive circuits. A common cathode package can be used for diodes D1 and D2. Less number of components can configure the circuit though two transformers are required. However, adopting two transformers may dilute the advantage of the flyback rectifier.

Integrated magnetics have been widely utilized to minimize component size and improve power density. The transformer in [15] features one set of primary and secondary windings placed on each of the outer legs of a three-legged core. This arrangement does not allow for flux cancellation in the center leg and is designed such that both transformers operate simultaneously. While this structure may help reduce input capacitor loss by distributing the ripple current, it requires precise tuning of the input current ripple and coupling, adding complexity to the design. The transformer in [16] adopts a similar physical layout as in [15], but with the windings arranged to enable flux cancellation in the center leg. Coupling between the windings allows for a reduction in input current ripple; however, the design becomes more complex, since the magnetic coupling affects zero-voltage switching (ZVS) and active clamping. The transformer in [17] also uses a similar configuration, but with inversely coupled secondary windings. In this case, careful design of the secondary-side coupling is required to meet specific output current ripple requirements, and the inverse coupling may introduce unwanted circulating currents or EMI if not properly managed. The transformer in [18] adds an inductor winding on the center leg, in addition to the transformers on the outer legs, to further control output current ripple. However, this increases the core’s magnetic complexity and can make magnetic design and analysis more challenging. A detailed comparison of these integrated magnetics structures is provided in Table I.

This paper proposes a non-coupled integrated transformer for a two-phase interleaved flyback rectifier as shown in Fig. 2(a). The proposed winding structure is shown in Fig. 2(b) where Nij is the number of turns between terminals i and j. Half of N12 and N34 are wound at each outer leg with opposite polarity, so as to allow for flux cancellation at the center leg. On the other hand, N56 and N78 are also wound with opposite polarity on the center leg, and allows for the transformer N12 and N34 to not operate during its operation. Therefore, the proposed winding structure employs zero coupling coefficient between the two transformers N12-N34 and N56-N78.

The zero-coupling coefficient between the two separate transformers simplifies implementation of rectifiers and transformer design. The operation of the rectifiers in each interleaved phase still follows that of a conventional flyback converter, preserving simplicity in control and timing. The design is also highly dependent on the duty cycle, an easily adjustable parameter in circuit design. The converter in this study aims to operate in DCM in order to allow for easier implementation of PFC and to conform to the standards of USB PD 3.1.  It also realizes independent phase operation, allowing the circuit to function in one-phase mode if desired, thereby enhancing flexibility in circuit operation.

The rest of this paper is organized as follows: Section II presents an operational analysis of a two-phase interleaved flyback rectifier with the integrated transformer, along with its design method. Section III describes the size reduction effect of the transformer. Sections IV and V discuss the results of the simulation and experimental verification respectively, and Section VI presents the conclusions of this paper.

# II. Design Method of the Proposed Transformer

## A. Operation Analysis of the Converter and Transformer

The proposed integrated transformer was designed for a two-phase interleaved flyback PFC rectifier operating in discontinuous conduction mode (DCM). The switches (S1, S2 in Fig. 2 (a)) operate with a fixed duty cycle D and  a 180-degree phase shift.

Half of N12 and N34 are wound at each outer leg with opposite polarity. Moreover, N56 and N78 are also wound with opposite polarity at the center leg. The transformer utilized an EE Core with a center leg that has a cross sectional area twice its outer leg, as per commercial standard, causing N12 = 2 N56 and N34 = 2 N78. The magnetizing inductances for both N12-N34 and N56-N78 were set to be the same. The saturation magnetic flux in each separate transformer during each phase is expected to be consistent.

The operational principle of the rectifier with the proposed transformer is illustrated in Fig. 3. Additionally, Fig. 4 presents the key waveforms of the six distinct operation modes. The rectifier is designed with duty cycle D ≤ 0.5 nVoVin,max, with n as the turns ratio, Vo as the output voltage, and Vin, max as the peak value of the input voltage. This condition ensures the magnetizing current of one phase returns to zero before the other phase peaks. This duration is defined as the discharge time tdischarge and is given in terms of circuit parameters by (xx):

tdischarge=Vin, maxnVODTs   (xx)

with Ts as the switching period.

The condition for the duty cycle imposed in this operational analysis is further explained in Section II-B. The proposed transformer is not limited to this condition. However, analysis of other operating modes lies beyond the scope of this paper.

To describe the magnetic state of the core, the superposition principle is used. The magnetic flux density component B’ is generated by the positive cycle transformer defined as N12-N34.  The magnetic flux density component B'' represents the component generated by the negative cycle transformer (N56-N78). The total flux in any leg is the algebraic sum of these components. This superposition determines the peak flux density required for the core design, which is analyzed in detail in Section II-B.

Modes 1, 2, and 3 in Figs. 3(a)-(c) cover the operation during the positive AC input into the rectifier, while Modes 4, 5, and 6 in Figs. 3(d)-(f) describe the operation during the negative AC input. For the following modes, B’o1, B’o2, and B’c represent the magnetic flux densities in the respective left, right, and center legs of the core caused by N12-N34, respectively. On the other hand, B’’o1, B’’o2, and B’’c represent the magnetic flux densities in the respective left, right, and center legs of the core caused by N56-N78, respectively, which are detailed visually in Fig. 5.

The following happen during the six modes of the condition described:

- Mode 1 (t1  < t < t2, Fig. 3(a)): At time t1, switch S1 turns on and S2 remains off. The transformer N12-N34 begins storing energy in its magnetizing inductance. Its magnetizing current iLm1 and the corresponding flux densities in the outer legs B’o1 and B’o2 linearly increase from zero. Simultaneously, N56-N78 completes its discharge phase, with its magnetizing current iLm2 and secondary diode current iD2 decreasing to zero at t2.

- Mode 2 (t2 < t < t3, Fig. 3(b)): Mode 2 starts right after N56-N78 completes its discharging phase, which means its associated currents iD2, iLm2, and flux densities B’’c, B’’o1, and B’’o2 reached zero. Now, with S1 still on,  N12-N34 continues storing energy until iLm1 reaches its peak value at t3. The peak primary current in the positive cycle iS1, pk is given by (1):

iS1,  pk =vin⋅DTsLm1#1

where vin is the input voltage, D is the duty cycle, Ts is the switching period, and Lm1 is the magnetizing inductance of N12-N34. Consequently, the flux densities B’o1 and B’o2 also reach their peak values B’o pk which are given by (2):

Bo'pk=Lm1iS1, pkN12Ao#2

with N12 as the number of turns in the primary side of the active transformer and Ao is the cross-sectional area of the outer-leg, where N12-N34 is wound.

- Mode 3 (t3 < t < t4, Fig. 3(c)): Mode 3 starts right after N12-N34 finishes charging. At t3, Switch S1 turns off, initiating the discharge phase of transformer N12-N34. The magnetizing current iLm1, diode current iD1, and flux densities B’o1 and B’o2 begin to decrease linearly. Transformer N56-N78 remains inactive in this period.

In Modes 1-3, the magnetomotive force and magnetic resistance of both outer legs are equal. The magnetic flux in the center leg is canceled. This causes no electromotive force generated in N56-N78. Therefore, the transformer formed by the outer legs operates independently from the transformer in the center leg. The integrated transformer is equivalent to that of Fig. 5(b) and its equivalent magnetic circuit is shown in Fig. 6(a).

- Mode 4 (t4 < t < t5, Fig. 3(d)): Mode 4 operates similarly to the positive cycle. Switch S2 turns on at t4. Transformer N56-N78 stores energy in its magnetizing inductance. Current iLm2 and B’’o1, B’’c, and B’’o2 increase linearly. Simultaneously, N12-N34 completes its discharge. Both iLm1 and iD1 decrease to zero at t5.

- Mode 5 (t5 < t < t6, Fig. 3(e)):

- Mode 5 begins after the discharge of N12-N34. Switch S2 remains on, N56-N78 continues storing energy until iLm2 reaches its peak value at t6. The peak primary current in the negative cycle iS2, pk is given by (3):

iS2,  pk =vin⋅DTsLm2#13

where Lm2 is the magnetizing inductance of N56- N78. Flux densities B’’o1, B’’o2, and B’’c also reach their peak values, given by (4):

Bc''pk=Bo''pk=Lm2iS2, pkN56Ac#24

with N56 as the number of turns in the primary side of the active transformer and Ac is the cross-sectional area of the outer-leg, where N56-N78 is wound.

- Mode 6 (t6 < t < t7, Fig. 3(f)): Switch S2 turns off, initiating the discharge of N56-N78. Currents iLm2, iD2, and B’’o1, B’’c, and B’’o2 decrease linearly. Transformer N12-N34 remains inactive.

For modes 4-6, which is the negative cycle, since the magnetic resistance of both outer legs is equal, the magnetic flux is divided equally into both outer legs. Furthermore, since the number of turns of both outer legs is equal and wound in an opposite polarity, the electromotive force generated in N12-N34 is canceled. Therefore, the transformer formed by the center leg operates independently from the transformer in the outer leg, and the integrated transformer is equivalent to that of Fig. 5(c). The equivalent magnetic circuit for this case is shown in Fig. 6(b).

[deleted]≤ nVoVin,max

tdischarge=Vin, maxnVODTs

The proposed winding structure achieves zero magnetic coupling between the two transformers. Consequently, the distinct magnetic circuits share a single core without magnetic interference. Each transformer maintains independent operation despite the physical integration. This structure eliminates the need for two discrete magnetic components. As a result, the total volume of the magnetic components is significantly reduced, with the degree of reduction explained in Section IV.

B. Saturation Magnetic Flux Density Analysis

The circuit operation is classified into three patterns determined by the duty cycle D and the boundary condition D ≤ 0.5 nVoVin,max. This classification, illustrated in Fig. 7, arises because the integrated single-core structure permits simultaneous magnetizing current flow in multiple active windings, resulting in distinct flux superposition compared to conventional discrete transformers. The simultaneous flow of magnetizing current can be controlled by tdischarge as shown in (xx) which depends on D. Consequently, the saturation magnetic flux density Bsat is a critical parameter for the transformer geometry. The flux density in each leg is examined further using the principle of superposition, with the expressions of the superimposed flux as shown in (3)-(5).

Bo1=Bo1'+Bo1''#3Bo2=Bo2'-Bo2''#4Bc=Bc''#5

The flux density is examined further using the principle of superposition, with the expressions of the superimposed flux as shown in Fig. 4 in (3)-(5).

Bo1=Bo1'+Bo1''#3Bo2=Bo2'-Bo2''#4Bc=Bc''#5

[deleted]

[Case1 : D < 0.5 and D ≤ 0.5 nVoVin,max]: The waveforms shown in Fig. 4 represent the circuit operation during case 1. Superimposing the flux densities with each other will result in the magnetic flux density waveforms in Fig. 7(a). The outer legs are not capable of magnetic flux cancellation, as shown in Fig. 7(a). However, the peak fluxes are seen in all legs. If the magnetizing current of one phase reaches zero before the magnetizing current of other flyback converter reaches its peak, then the saturation magnetic flux density of the core across all the phases, Bmax, is equal to B’max.  This condition is satisfied when D is as expressed by (6) in addition to being less than 0.5.

D≤0.5nVoVin,max#6

[Case2 : D < 0.5 and D > 0.5 nVoVin,max: The circuit can be analyzed similarly to case 1, and Fig. 7(b) shows the waveforms of the superimposed fluxes of the integrated transformer. Here, one transformer does not discharge before the other fully charges, or its magnetizing current reaches its peak. The outer legs are not capable of flux cancellation as shown here. The saturation flux density across all phases can be derived as following. At t2, B’o1 = B’max. Moreover, B’’o1 (t2) = B’o1 (t5), and is expressed in (7).

Bo1''t2=Bo1't5=Bmax'-Bmax't6-t2(t5-t2)#7

With t5 = 0.5Ts + t2 = 0.5Ts + DTs  and t2 = DTs. t6 can be obtained by examining the increase and decrease of of the magnetizing inductance current during charge and discharge of the transformer. The peak can be rewritten as in (8):

iLm1, pk=Vin, maxLmDTs=nVoLmt6-t2#8

The superimposed Bmax can now be found. In this case, (6) is not satisfied, and this results in Bmax in the left outer leg exceeding B’max in the center leg, as expressed in (9).

Bmax=2-0.5nVoVin,maxDBmax'#9

[Case3 : D > 0.5]: Fig. 7(c) shows the waveforms of the superimposed fluxes of the integrated transformer. In this case, Bmax in the left outer leg always exceeds B’max, and Bmax is expressed by (10), and is obtained by analyzing the flux waveforms in a similar way to case 2.

Bmax=1+D-0.5DBmax'#10

Because the flyback is interleaved, it is expected that the rectifier will primarily operate at D < 0.5 (case 1 or 2) [cite]. The analysis identifies the peak flux conditions across all cases to ensure the core dimensions prevent saturation depending on the desired operation.

C. Design Method of the Proposed Integrated Transformer

A turns ratio n in consideration of the maximum rated voltages of the switches used. A duty cycle D is selected to decide an operating pattern. B’max is calculated from the chosen D. Afterwards, the magnetizing inductance, Lm1 = Lm2 = Lm in this paper, is calculated from the circuit specifications. A core is chosen using the area-product method. The number of turns is calculated using B’max and the peak primary side current Ipri, pk as shown in (11) and (12).

N34≥LmIpri, pknAmoB'max,N12=nN34#11

N78≥LmIpri, pknAmcB'max,N56=nN78#12

The magnetic resistances of the outer legs Rmo and the central leg Rmc are obtained from a magnetic circuit analysis of the integrated transformer in its separated components. The transformer N12-N34 is only made of the outer legs as shown in Fig. 8(b) while that of N56-N78 is made of all the legs. The magnetizing inductances implied by N12-N34 and N56-N78 are shown in (13) and (14) respectively.

Lm=N1222Rmo#13

Lm=N562Rmc+0.5Rmo#14

Rmo and Rmc is obtained from (15) and (16).

Rmo=N1222Lm#15

Rmc=2N56-N122N56+N124Lm#16

The EE core configuration for the proposed integrated transformer, which includes the effective cross-sectional area, effective magnetic path length, both determined from the core dimensions, and magnetic resistance is shown in Fig. 8.

Finally, an air gap was also inserted in each of the legs to be able to provide the desired resistance. Non-equal length air gaps were placed in the transformer. There are two cases, one in which the air gap in the outer legs lmo, air are longer than the air gap in the center leg lmc, air, and one in which lmc, air is longer than lmo, air. There is a small challenge in manufacturability at the moment, especially that there would be a need to physically perform shaving on some legs to get the desired air gap length.

[Case 1: lmo, air > lmc, air] Given the setup in Fig. 9a, the corresponding magnetic resistances of the outer and center legs are shown in (17) and (18) and can be solved for the corresponding air gap lengths as shown in (19) and (20).

Rmo=lmo-lmoair+lmcairμrμ0Amo+lmoairμ0Amo#17

Rmc=lmcμrμ0Amc+lmcairμ0Amc#18

lmoair=μ0μrAmoRmo-AmcRmc+lmcμr-lmoμr-1#19

lmcair=μ0AmcRmc-lmcμr#20

[Case 2: lmc, air > lmo, air] Given the setup in Fig. 9b, the corresponding magnetic resistances of the outer and center legs are shown in (21) and (22) and can be solved for the corresponding air gap lengths in a similar way as shown in (23) and (24).

Rmo=lmoμrμ0Amo+lmoairμ0Amo#21

Rmc=lmc-lmcair+lmoairμrμ0Amc+lmcairμ0Amc#22

lmoair=μ0AmoRmo-lmoμr#23

lmcair=μ0μrAmcRmc-AmoRmo+lmoμr-lmcμr-1#24

# III. Simulation Verification of Proposed Design

A. Simulation Verification of the Proposed Design Method

To ensure the validity and operation of the proposed integrated transformer, a simulation of the two-phase interleaved flyback rectifier was first performed in PLECS Standalone 4.9.4, with the diagram shown in Fig. 10(a). Clamped RCD Snubber circuits (Csn1, Dsn1, Rsn1, and Csn2, Dsn2, Rsn2) were also added to the circuit for protection against surge voltage [15]-[16] caused by the leakage inductance and the switch capacitance at the turn-off time of S1 and S2. An integrated transformer, shown in Fig. 10(b), was also designed using the proposed method. The circuit and transformer were designed with the condition given by (6), and will thus be verified in this study. The specifications of the two-phase interleaved flyback rectifier and transformer for the simulation verification are shown in Tables II and III respectively. The prototype circuit has no control stage for the output voltage yet since this study only focuses on a proof of concept first. The duty cycle is calculated using (25) [8].

D=2vin, pk0.5 LmPOTs#25

B. Simulation Results

Figure 11 shows waveforms of from the simulation testing of the two-phase interleaved flyback rectifier with the proposed transformer at 240 W. Key waveforms of the circuit were observed from the simulation, which include vgs1 and vgs2, iLm1 and iLm2, and the fluxes Bo1, Bo2, and Bc . Similar waveforms to that of Fig. 4 have been observed. Flux  cancellation is observed in the center leg during the time that it is inactive, which then confirms the validity of the theoretical operation proposed in Section II. Moreover, the increase and decrease of fluxes were all linear, therefore confirming that there is indeed no magnetic coupling interference. The effects of the resonance between the leakage inductance and parasitic capacitance can however be seen in the waveforms. Saturation was also not observed. Therefore, from simulation, the two flyback converters N12, N34, S1, D1, CO, VO and N56, N78, S2, D2, CO, VO can operate independently without magnetic coupling interference, as coupling was eliminated between the flyback transformers through the  winding structure.

# IV. Size Reduction Effect of the Proposed Transformer

This section examines the size reduction effect by the integrated transformer. For this study, the conventional transformer is assumed to be the two separate transformers that facilitate isolation between the primary and secondary side. A representation is shown in Fig. 12 and Fig. 13, with windings.

The volume is compared by using the area-product (AP) method. Especially that the AP method relates the circuit specifications, such as the current density and magnetizing inductance, to the geometric details of the transformer, it can be a suitable way to determine the volume of the transformer. The AP method utilizes the product between the core cross-sectional area, Ac and the window-area, Aw, and is a fundamental core-se for the application [21]. The EE-core was redesigned in such a way such that Aw and Ac are squares to allow for simpler analysis, as shown in Fig. 14 [22]. Here, Ac is twice the outer leg. This leads to N12 and N34 having twice as many turns as N56 and N78, respectively. The variables and parameters to be used in this analysis is shown in Table 4.

For the conventional transformer, Ac is expressed in (26). Here, Np is the number of turns wound on the center leg of the conventional transformer.

Acconv.=LmIp,  pkBmaxNp#26

For the integrated transformer, Ac is expressed in (27).

Acinteg.=2LmIp,  pkBmaxN12=LmIp,  pkBmaxN56#27

For the conventional transformer, Aw can be expressed in terms of winding area, which takes a fraction of Aw, denoted by Ku, or the occupancy ratio, as seen in (29).

Awconv.Ku=NpIp,rmsJm+NsIs, rmsJm #28

Awconv.=NpJmKuIp,rms+Is,rmsn#29

For the integrated transformer, on the other hand, taking into account that the core window has half the total turns of the transformer N12, N34, the Aw can be expressed as seen in (31).

Awinteg.Ku=N56Ip,rmsJm+N78Is,rmsJm +12N12Ip,rmsJm+N34Is,rmsJm                     (30)

Awconv.=2N56JmKuIp,rms+Is,rmsn#31

The area-product of the conventional and integrated transformers are shown in (32) and (33) respectively using (26) – (31).

APconv.=Awconv. ⋅Acconv. = LmIp,peakIp,rms+Is,rmsnBmJmKu #32

APinteg.=Awinteg. ⋅Acinteg. =2LmIp,peakIp,rms+Is,rmsnBmJmKu=2APconv.#33

The volume of the core shown in Fig. 11 can be expressed using the expression shown in (34), where x is a constant that denotes the ratio Ac/Aw. The total volume of the conventional transformer is twice one transformer, since two cores comprise this transformer.

Vol.=2AP34x34+2x14+x-14#34

The rms expressions of the primary and secondary side current are obtained using are given in (35) and (36) [8].

Ip, rms=Vin,max DLmfsD36#34

Ip, rms=Vin,max DLmfs5nD39π⋅Vin, maxVO#35

Table VI
Specifications of the prototype 
two-phase interleaved flyback rectifier

| Item | Symbol | Value, component |
| --- | --- | --- |
| AC input voltage | vin | 100 – 240 Vrms |
| Max. Rated Power | Po | 240 W |
| Output voltage | Vo | 48 V |
| Sw. frequency | fs | 100 kHz |
| Bridge diode | BD | CD-DF408S |
| Input capacitor | Cin | 1 μF |
| Switch | S1, S2 | STF15N80k5 |
| Secondary diode | D1, D2 | STPSC30H12CWL |
| Output capacitor | Co | 2 x 1 mF (parallel) |
| Snubber capacitor | Csn1, Csn2 | 2.2 nF |
| Snubber resistor | Rsn1, Rsn2 | 30 kΩ |
| Snubber diode | Dsn1, Dsn2 | VS-E7MH0112-M3/I |

- Table VII
Prototype integrated transformer specifications

| Item | Symbol | Value |
| --- | --- | --- |
| Mag. inductance (Target) | Lm | 82.8 μH |
| Mag. inductance of N12 | Lm1 | 86.1 μH |
| Mag. inductance of N56 | Lm2 | 75.1 μH |
| Turn ratio | n | 3 |
| Magnetic core | T | OP44721EC |
| Number of turns | N12 : N34 | 24 : 8 |
|  | N56 : N78 | 15 : 5 |
| Center Leg Air Gap | lgc | 0.650 mm |
| Outer Leg Air Gap | lgo | 0.500 mm |
| Coupling coeff. of N12, N34 | kN12, N34 | 0.970 |
| Coupling coeff. of N56, N78 | kN56, N78 | 0.979 |
| Coupling coeff. of N12, N56 | kN12, N56 | 0.0005 |

Table V
Volume analysis of the conventional and proposed transformer using the chosen core OP44721EC.

| Item | Symbol | Conventional | Integrated |
| --- | --- | --- | --- |
| Mag. Inductance | Lm | 82.8 μH |  |
| Turn ratio | n | 3 |  |
| Primary side pk. current | Ip, pk | 8.02 A |  |
| Primary side RMS current | Ip, rms | 2.25 A |  |
| Secondary side RMS current | Is, rms | 6.88 A |  |
| Maximum flux density | Bmax | 0.3 T |  |
| Maximum current density | Jmax | 5 A/mm2 |  |
| Occupancy ratio | Ku | 0.3 |  |
| Area Product | AP | 6703.21 mm4 | 13406.42 mm4 |
| Core Volume | Vol. | 10313.57 mm3 | 8672.64 mm3 |

Using (31), (32), and (33) with the actual dimensions of the core used in the experimental verification, a 19% volume reduction has been achieved by the proposed design, with the comparison shown in Table V. The maximum duty set was 0.47, and was used in the calculation.

# V. Experimental Verification of Proposed Design

A. Prototype Evaluation Circuit and Prototype Transformer

A prototype two-phase interleaved flyback rectifier, shown in Fig. 15(a) operating in DCM was built to verify the operation of the integrated transformer (diagram s. The prototype integrated transformer, shown in Fig. 15(b), was also designed using the proposed method.  The specifications of the two-phase interleaved flyback rectifier and transformer are shown in Tables VI and VII respectively. The coupling coefficient between the two flyback transformers, which is the coupling coefficient between N12 and N56, is 0.0005, showing that the coupling was almost eliminated, and with the magnetizing inductance of the prototype close to the target. The number of turns were adjusted so that the target inductance is met as close as possible.

D. Experimental Results

Fig. 16(a) shows the 100 W experimental waveforms at 4 ms/div. from the two-phase interleaved flyback rectifier with the proposed transformer, demonstrating power factor correction.  Fig. 16(b) presents 2 μs/div. waveforms, with the off-times of S1 and S2 indicated. The current is1 is zero during the off-time of S1 and is2 is zero during the off-time of S2. No current is induced in either switch except during the resonance between the leakage inductance and the parasitic capacitance of the switch. During the inactive period of the transformer, for example, between the start of the off-time of S1 and the start of the off-time of S2, the secondary diode side current is zero. Moreover, after iD1 and iD2 reached zero when S1 and S2 switched off, the currents remained at zero. Lastly, a linear increase and decrease of iS1, iS2, iD1, and iD2 was observed throughout the circuit operation. Therefore, the two flyback converters N12, N34, S1, D1, Co, Vo and N56, N78, S2, D2, Co, Vo operate independently without magnetic coupling interference, as the coupling was eliminated between the flyback transformers. The same phenomenon was also observed at 240 W two-phase drive, the highest load for this experiment, as shown in Fig. 16(c) and 16(d).

Single-phase drive at lower power is also feasible with this transformer without magnetic interference. In the 4 μs/div. waveform shown in Fig. 16(d) at 32.5 W output of single-phase drive of S1, it can be confirmed that only the flyback converter N12, N34, S1, D1, Co, and Vo are operating, since both iS2 and iD2 are zero during the whole operation.

VI. Conclusion

In this paper, an integrated transformer was successfully designed for a two-phase interleaved flyback rectifier for USB PD 3.1 applications. The proposed winding structure for the integrated transformer prevents coupling between the two flyback transformers, allowing the two flyback converters to operate independently in each phase without magnetic interference. A very low coupling coefficient of 0.0005 was achieved with the prototype. Independent flyback circuit operation without magnetic interference due to coupling was verified through simulation and experiment using the designed transformer and an evaluation circuit up to 240 W. Because of the proposed design, two transformers are integrated into one component, enabling the downsizing of magnetic components in the circuit. The extent of miniaturization was found to be about 19% compared to the conventional transformer.

References

- A. Medina-Garcia, J. Cruz-Cozar, D. P. Morales-Santos, N. Rodriguez, and M. Schlenk, “A High-Efficiency Isolated Wide Voltage Range DC-DC Converter Using WBG Devices,” IEEE Access, vol. 10, pp. 84818-84825, Aug. 2022.

- USB-IF PR, "USB Promoter Group Announces USB Power Delivery Specification Revision 3.1," 2021. [Online]. Available: https://www.usb.org/sites/default/files/2021-05/USB%20PG%20USB%20PD%203.1%20DevUpdate%20Announcement_FINAL.pdf

- B. C. Kim, K. B. Park, C. E. Kim, B. H. Lee, and G.W. Moon, “LLC resonant converter with adaptive link-voltage variation for a high-power-density adapter,” IEEE Trans. Power Electron., vol. 25, no. 9, pp. 2248-2252, May 2010.

- Y. C. Lin, F. C. Lee, Q. Li, X. Huang, and Z. Liu, “A novel AC-to-DC adaptor with ultra-high power density and efficiency,” in IEEE Applied Power Electronics Conference and Exposition, 2016, pp. 1853-1860.

- M. Li, Z. Ouyang, and M. A. E. Andersen, “Analysis and optimal design of high-frequency and high-efficiency asymmetrical half-bridge flyback converters,” IEEE Trans. Indus. Electron., Vol. 67, no. 10, pp. 8312-8321, Nov. 2019.

- X. Huang, J. Feng, W. Du, F. C. Lee, and Q. Li, “Design consideration of MHz active clamp flyback converter with GaN devices for low power adapter application,” in 2016 IEEE Applied Power Electronics Conference and Exposition (APEC), 2016, pp. 2334 – 2341.

- J. Zhang, X. Huang, X. Wu, and Z. Qian, “A high efficiency flyback converter with new active clamp technique,” IEEE Trans. Power Electron., vol. 25, no. 7, pp. 1775-1785, Feb. 2010.

- J. Shin, S. Choi and B. Cho, “High-efficiency bridgeless flyback rectifier with bidirectional switch and dual output windings,” IEEE Trans. Power Electron., vol. 29, no. 9, pp. 4752-4762, Oct. 2013.

- S. Choi, J. Shin, J. Imaoka and M. Yamamoto, “Common-Mode Noise Reduction of Bridgeless Flyback PFC Rectifier with Balance Technique,” IEICE Tech. Rep. vol. 123, no. 119, pp. 67-72, 2023.

- K. T. Mok, Y. M. Lai, and K. H. Loo, “A single-stage bridgeless power-factor-correction rectifier based on flyback topology,” in 2011 IEEE 33rd International Telecommunications Energy Conference (INTELEC). 2011, pp. 1-6.

- J. Beak, J. Shin, P. Jang, and B. Cho, “A critical conduction mode bridgeless flyback converter,” 8th International Conference on Power Electronics-ECCE Asia, 2011, pp. 487-492.

- B. Shafer, “Interleaving contributes unique benefits to forward converters and flyback converters,” in Texas Instrument Power Supply Design Seminar, Topic, vol. 4, pp.1-32, 2004.

- L. P. Wong, Y. S. Lee, and D. K. W. Cheng, “A new approach to the analysis and design of integrated magnetics,” in Sixteenth IEEE Applied Power Electronics Conference and Exposition, 2001, pp. 1196-1202.

- S. Choi, J. Shin, J. Imaoka, and M. Yamamoto, “Voltage-Controlled Variable Inductor for Fixed-Frequency Critical Conduction Mode Operation,” IEEE Trans. Indus. Electron., vol. 70, no. 6, pp. 5707-5716, Aug. 2022.

- M. Ishihara, S. Kimura, W. Martinez and M. Yamamoto, "Analysis and design of passive components for interleaved flyback converter with integrated transformer," in 2015 IEEE Energy Conversion Congress and Exposition (ECCE), 2015, pp.5902-5909.

- M. Taheri, J. Milimonfared, H. Bayat, H. Riazmontazer and A. Noroozi, "Analysis and design of a new soft switching interleaved converter using an integrated transformer," in 2011 2nd Power Electronics, Drive Systems and Technologies Conference, 2011, pp. 98-103.

- T. Qian and B. Lehman, "Coupled Input-Series and Output-Parallel Dual Interleaved Flyback Converter for High Input Voltage Application,"  IEEE Trans. Power Electron., vol. 23, no. 1, pp. 88-95.

- Liang Yan, Dayu Qu and B. Lehman, "Integrated magnetic full wave converter with flexible output inductor," in Seventeenth Annual IEEE Applied Power Electronics Conference and Exposition, 2002, pp. 824-830.

- G. B. Koo, “Design guidelines for RCD snubber of flyback converters.” Fairchild Semiconductor, San Jose, CA, USA, Appl. Note AN-4147 Rev.1.1.0, 2006.

- W. McMurray, “Selection of snubbers and clamps to optimize the design of transistor switching converters.” IEEE Transactions on Industry Applications, vol. IA-16, no. 4, pp. 513 - 523, 1980.

- W. Colonel and T. McLyman, “Transformer and Inductor Design Handbook”, Boca Raton, FL, USA: CRC Press, 2004.

- S. Kimura, S. Aoto, M. Ishihara, J. Imaoka and M. Yamamoto, "Interleaved active clump forward converter with novel integrated magnetic components," in 2015 IEEE Energy Conversion Congress and Exposition (ECCE), 2015, pp. 6029-6036.

First A. Author (Fellow, IEEE) and all authors may include biographies if the publication allows. Biographies are often not included in conference-related papers. Please check the Information for Authors to confirm. Author photos should be current, professional images of the head and shoulders. The first paragraph may contain a place and/or date of birth (list place, then date). Next, the author’s educational background is listed. The degrees should be listed with the type of degree in what field, which institution, city, state, and country, and year the degree was earned. The author’s major field of study should be lowercase.

The second paragraph uses the preferred third person pronoun (he, she, they, etc.) and not the author’s last name. It lists military and work experience, including summer and fellowship jobs. Job titles are capitalized. The current job must have a location; previous positions may be listed without one. Information concerning previous publications may be included. The format for listing publishers of a book within the biography is: Title of Book (publisher name, year) similar to a reference. Current and previous research interests end the paragraph.

The third paragraph begins with the author’s preferred title and last name (e.g., Dr. Smith, Prof. Jones, Mr. Kajor, Ms. Hunter, Mx. Riley). List any memberships in professional societies other than the IEEE. Finally, list any awards and work for IEEE committees and publications.

Second B. Author, photograph and biography not available at the time of publication.

Third C. Author Jr. (Member, IEEE), photograph and biography not available at the time of publication.

Manuscript received Month xx, 2xxx; revised Month xx, xxxx; accepted Month x, xxxx.  (Corresponding author: Sihoon Choi).

Karl Raymond Dela Cruz Roque is with the Graduate School of Engineering, Nagoya University, Nagoya, 464-8601, Japan (e-mail: roque.karl.raymond.dela.cruz.g4@).

Hirasawa Haruki is with xxx, Japan (e-mail: xxx).

Sihoon Choi, Yu Yonezawa, Jun Imaoka and Masayoshi Yamamoto are with Institute of Materials and Systems for Sustainability, Nagoya University, Nagoya, 464-8601, Japan (email: choi.sihoon.w4@f.mail.nagoya-u.ac.jp; yonezawa.yu.c9@f.mail.nagoya-u.ac.jp; imaoka.jun.n9@f.mail.nagoya-u.ac.jp; yamamoto.masayoshi.n5@f.mail.nagoya-u.ac.jp).