Additionally, when it comes to interleaved flyback topologies, some utilize this magnetic coupling strongly. Commonly though, these require additional techniques to utilize the benefit. The interleaved flyback  Javon gfor clamp circuits to ensure this current sharing [22]. actively utilize this magnetic coupling to have additional benefits. there are many different approaches to the implementation of interleaved flyback converters. Some topologies try to achieve interleaved operation and ripple reduction using separate transformer structures paired with auxiliary filter magnetics for higher power operations (~1 kW). However, have increasingly focused on magnetic component miniaturization through core integration, as seen in conventional coupled designs that consolidate multiple transformer windings or coupled inductors onto a single core.. Although these conventional approaches effectively reduce overall magnetic volume or handle ripple currents, structures utilizing coupled integrated transformers inherently allow magnetic flux interaction between phases, which can lead to inter-phase dependency. They allow for advantage as long as it is combined with other control techniques, etc. herefore, implementing an integrated magnetic structure that achieves compact volumetric performance while strictly enforcing phase decoupling addresses a distinct gap in current converter design.