fResponse to the Reviewers’ Comments for TIA-2026-IPCC-0540

Manuscript Title: Design and Implementation of a Non-coupled Integrated Transformer for a Two-phase Interleaved Flyback PFC Rectifier

Oct 25, 2026

Dear Editor,

We would like to express our sincere appreciation to the reviewers for going through our response letter and first revision of our manuscript. We found their constructive and insightful comments really helpful in the preparation of the next revision. The manuscript was revised once again in accordance with the comments and suggestions of the reviewers:

- We thank the reviewer for going through and agreeing with our response letter.

- Pages 2 to 12 correspond to Reviewer #2’s comments.

- Pages 13 to 22 correspond to Reviewer #3’s comments.

We revised the manuscript accordingly to the reviewers’ comments. In particular, we added a summary of the literature review in the form of a table, gave insights on the PF and THD, and PFC capability of the topology with the proposed magnetic component, and also commented on the system-level efficiency. We also updated the manuscript to improve clarity, consistency, and minor errors.

We hope the manuscript is now acceptable for publication.

Best Regards,

Sihoon Choi

Corresponding author,

Institute of Materials and Systems for Sustainability, Nagoya University

Email: choi.sihoon.w4@f.mail.nagoya-u.ac.jp

# To Reviewer 2

We authors appreciate your valuable comments. The revised parts are indicated by red texts in the manuscript.

Comments to the Author

The analytical explanation of the inherent DCM PFC mechanism is now clearer. However, the experimental verification remains insufficient. Please report measured PF and input-current THD, preferably at representative operating points.

We sincerely thank the reviewer for the valuable feedback and for acknowledging the clarity of our analytical explanation. We completely agree that the power factor PF and input-current THD are critical metrics for PFC flyback rectifiers. However, because the inherent DCM PFC mechanism is already a well-known and established characteristic, our physical prototype was constructed without a grid-side EMI filter to focus on validating the magnetic decoupling of the proposed integrated core. We would also like to emphasize that the proposed magnetic integration does not alter the fundamental electrical principle of the converter. The inherent DCM PFC mechanism remains identical to that of a conventional topology utilizing discrete transformers. To rigorously demonstrate this, we have conducted a comprehensive simulation in PLECS Standalone 5.0 with a standard differential-mode EMI filter (Lf=100 μH, Cf=1 μF) to obtain an accurate estimate.

As shown in the table below, we also compared the proposed integrated transformer against a conventional interleaved flyback baseline using two discrete transformers with matched magnetizing inductance Lm of 82.8 μH and turns ratio n=3. The topology with the proposed magnetic component can preserve PFC, exhibiting PF and input-current THD performance that closely matches the conventional discrete setup across the extremes of the operating range, maintaining PF >0.95 at all representative points.

We have added PFC waveforms as well in various operating points for a visual reference for the reviewers. We have revised Section III completely to reflect these changes as indicated below.

(a)                                                                           (b)

Key PFC Waveforms of the proposed two-phase interleaved flyback rectifier showing vin, vo, iin, iS1 and iS2, and vds1 and vds2 with vin, Vout, and PO of: (a) 100 Vrms, 48 V, and 240 W and (b) 240 Vrms, 48 V, and 240 W.

| Before revision | After revision |
| --- | --- |
| N/A | (in Section III-B, after the 1st paragraph) Furthermore, to substantiate the PFC capability of the proposed integrated structure, a performance analysis was conducted across several operating points. For reference, the proposed design was compared against a conventional two-phase interleaved flyback rectifier utilizing two discrete transformers with an identical n = 3 and Lm=82.8 μH. To extract clean grid-side AC waveforms and accurately evaluate harmonic distortion, a standard differential-mode EMI filter withLf=100 μH, Cf=1 μF was incorporated into the simulation model. As summarized in Table IV, the proposed non-coupled integrated transformer maintains a high PF of >0.95 and low current THD across various input voltages and load conditions. Crucially, the fundamental decoupling of the magnetic structure ensures that its PF and THD performance remains nearly identical to that of the conventional discrete implementation and additionally validates the claims of inherent PFC operation in DCM in Section II. To provide a clear visual reference of this capability, the simulated PFC waveforms at the operating points of vin, VO, and PO of 100 Vrms, 48 V, and 240 W, and 100 Vrms, 48 V, and 240 W are shown in Fig. 13(a) and Fig. 13(b) respectively. (a) (b) Fig. 13. Key PFC Waveforms of the proposed two-phase interleaved flyback rectifier with vin, Vout, and PO of: (a) 100 Vrms, 48 V, and 240 W and (b) 240 Vrms, 48 V, and 240 W. |

The added time-domain analysis satisfactorily explains the variation over one line cycle. Nevertheless, the manuscript should also clarify the variation of peak switch current and peak flux density when the RMS input voltage changes from 100 to 240 V, including the corresponding duty-cycle adjustment. Please identify the actual worst-case operating condition for switch current and core flux density.

We sincerely thank the reviewer for the valuable feedback and for acknowledging the clarity of our revised theoretical analysis. We completely agree that clarifying the component stresses across the universal input voltage range (100 Vrms to 240 Vrms) provides a more comprehensive understanding of the operating limits of the converter as a whole.

To address the variation in peak switch current and peak flux density, we must look at how the duty cycle D adjusts with the RMS input voltage Vin. In a two-phase interleaved flyback PFC rectifier operating in DCM delivering constant output power Po under the same magnetizing inductance Lm and same switching period Ts, D is inversely proportional to the peak input voltage Vinmax governed by (30) in the manuscript, which directly follows from [6]:

D=2Vinmax0.5LmPoTs

When Vin increases from 100 Vrms to 240 Vrms, if a control loop is added, it will naturally decrease D to maintain the same power transfer. If the adjusted D is substituted into (2) and (4) in the manuscript, the absolute peak switch current  ISpkmax which is at the peak of the AC line cycle, is given as:

ISpkmax=VinmaxLmDTs=VinmaxLm2Vinmax0.5LmPoTsTs=20.5PoTsLm

As shown, the Vinmax term mathematically cancels out. Moreover, from (3) and (5) in the manuscript, it follows that ISpkmax and the absolute peak core flux density Bpk(max)remain theoretically constant across the entire 100 V to 240 V input range.

However, the worst-case operating condition for the current and the magnetic core is still the lowest input voltage at 100 Vrms. While the peak current remains constant, D reaches its absolute maximum at 100 Vrms. Consequently, the RMS currents in the switches and transformer windings are maximized at 100 Vrms, leading to the highest conduction losses and winding losses. Factoring in the practical drop in efficiency at lower line voltages, the peak current will be slightly higher at 100 Vrms to compensate for these losses, cementing it as the worst-case condition for current stress and core saturation. Conversely, the highest input voltage 240 Vrms is the worst-case condition for the switch voltage stress Vdsmax. We have added a new paragraph to the end of Section II-A to explicitly clarify this dynamic across the universal input voltage range and clearly identify the worst-case conditions for the readers.

The simulation and experimental waveforms in Fig. 12 and 17 (Fig. 18 in the revised manuscript) respectively were taken under the worst-case operating conditions and show that the converter can operate without any problem with the integrated transformer. This is also clarified in the paper as follows.

| Before revision | After revision |
| --- | --- |
| N/A | (After Section II-B). C. Worst-Case Operating Conditions To properly evaluate the converter and size the components across the universal input range vin 100 Vrms to 240 Vrms, it is critical to identify the worst-case operating conditions. For both the switch current and the magnetic core, the lowest vin of 100 Vrms represents the practical worst-case scenario, whereas the highest vin of 240 Vrms dictates the worst-case switch voltage stress vdsmax. This distinction is fundamentally driven by the necessary duty-cycle adjustment. In a two-phase interleaved flyback PFC rectifier operating in DCM and delivering constant output power Po, D is inversely proportional to Vinmax, which directly follows from [6] and is expressed in (8): D=2Vinmax0.5LmPoTs (8) From (2) and (4), using (8), the absolute peak switch currents IS1pk (max), IS2pk (max) which are at the peak of the AC line voltage cycle, remain constant across the entire input range. It also follows from (3) and (5) that the absolute peak magnetic flux densities B'o1pk (max), B''o2pk (max) and B''cpk(max) remain constant across the entire vin of 100 Vrms to 240 Vrms. However, while the peaks remain constant, D reaches its absolute maximum at the lowest vin. Consequently, the RMS currents in the switches and transformer windings are maximized at the lowest vin. Furthermore, accounting for the degradation of converter efficiency at lower line voltages, the system must draw greater input power to sustain the nominal Po. This requirement inherently elevates the actual peak currents at vin of 100 Vrms, establishing it as the most severe operating condition for both current stress and magnetic core saturation. D. Transformer Design Method …. |

The expanded literature survey is appreciated. However, the requested comprehensive comparison would be considerably clearer in tabulated form.

We sincerely thank the reviewer for the positive feedback regarding the expanded literature survey and for this highly constructive recommendation. We agree that a tabulated format significantly improves the clarity and accessibility of the state-of-the-art comparison. In accordance with the suggestion, we have introduced a comprehensive comparison table in Section I-A of the revised manuscript. We present the table here as follows.

| Before revision | After revision |
| --- | --- |
| (in Section I, 11th Paragraph) Structural decoupling approaches have been explored to bypass these circulating currents and accommodate the variable load requirements of USB PD, offering an alternative to control-based phase shedding. Some approaches design decouples phases by separating the flux paths using 3-D orthogonal flux core geometries [34], [35]. However, these custom cores constrain commercial manufacturability and increases fabrication costs. Another proposed solution is an integrated nonlinear coupled inductor designed to decouple four phases at light loads [18]. Achieving this variable coupling requires customized, multi-piece L- and T-shaped core assemblies with manually configured geometries, introducing tight assembly tolerances and manufacturing complexity. | (in Section I, 11th Paragraph) Structural decoupling approaches have been explored to bypass these circulating currents and accommodate the variable load requirements of USB PD, offering an alternative to control-based phase shedding. Some approaches design decouples phases by separating the flux paths using 3-D orthogonal flux core geometries [34], [35]. However, these custom cores constrain commercial manufacturability and increases fabrication costs. Another proposed solution is an integrated nonlinear coupled inductor designed to decouple four phases at light loads [18]. Achieving this variable coupling requires customized, multi-piece L- and T-shaped core assemblies with manually configured geometries, introducing tight assembly tolerances and manufacturing complexity. To clarify the operational trade-offs among these existing approaches, a comprehensive comparison of state-of-the-art interleaved flyback topologies and their magnetic configurations is summarized in Table I. |

| Before revision | After revision |
| --- | --- |
| N/A | (In Section I, at the bottom of Page 2) |

Although the quality of the experimental waveforms has improved, the waveform traces should be thickened further to enhance visibility and readability.

We thank the reviewer for pointing this out. We completely agree that the visibility and readability of the experimental results are paramount. We have revised all experimental waveform figures by thickening the traces once again and improving the overall contrast for maximum clarity. Please see the updated waveforms below, which have now been incorporated into the revised manuscript.

| Before revision | After revision |
| --- | --- |
| (in Section V, Fig. 17) (b) (c) (d) Fig. 17. Experimental waveforms of the proposed two-phase interleaved flyback rectifier at (a) and (b): 100 W, two-phase AC drive; (c): 240 W, two-phase AC drive; (d): 32.5 W, single-phase AC drive. For (a) and (b): vin: 50 V/div., Vo: 10 V/div., vds1, vds2: 100 V/div., iS1, iS2: 1 A/div., iD1, iD2: 5 A/div.; For (c): vin: 50 V/div., Vo: 10 V/div., vds1, vds2: 100 V/div., iS1, iS2: 2 A/div., iD1, iD2: 5 A/div.; For (d): vin: 50 V/div., Vo: 20 V/div., vds1, vds2: 100 V/div., iS1, iS2: 2 A/div., iD1, iD2: 2 A/div. | (in Section V, Fig. 17) (b) (c) (d) Fig. 17. Experimental waveforms of the proposed two-phase interleaved flyback rectifier at (a) and (b): 100 W, two-phase AC drive; (c): 240 W, two-phase AC drive; (d): 32.5 W, single-phase AC drive. For (a) and (b): vin: 50 V/div., Vo: 10 V/div., vds1, vds2: 100 V/div., iS1, iS2: 1 A/div., iD1, iD2: 5 A/div.; For (c): vin: 50 V/div., Vo: 10 V/div., vds1, vds2: 100 V/div., iS1, iS2: 2 A/div., iD1, iD2: 5 A/div.; For (d): vin: 50 V/div., Vo: 20 V/div., vds1, vds2: 100 V/div., iS1, iS2: 2 A/div., iD1, iD2: 2 A/div. |

# To Reviewer 3

We authors appreciate your valuable comments. The revised parts are indicated by red texts in the manuscript.

Comments to the Author

This paper analyzes the design and implementation of an interleaved Flyback converter based on magnetic integration. My comments are as follows:

- The reviewers suggested including efficiency tests and comparing the results with other similar state-of-the-art topologies, but the authors did not adopt this recommendation.

- The efficiency curves provided in the authors' response are difficult to read due to the excessively wide range of the vertical axis. Furthermore, for an interleaved flyback converter, the prototype's efficiency is merely 85%—a figure far too low to meet the requirements for power electronic converters. While component selection certainly plays a significant role, the efficiency should not be this poor; such results suggest a fundamental flaw in the coupled transformer design or indicate that the topology itself may be incapable of achieving high efficiency.

We sincerely thank the reviewer for their critical feedback. Because Comments 1 and 2 both address the efficiency of the converter, we have combined our responses to address these concerns cohesively.

First, we apologize for the poor readability of the efficiency curve provided in our previous response. We have rescaled the vertical axis of the plots to closely match the peak efficiency, removing unnecessary empty space to clearly display the data range. As measured using a Hioki PW8001 Power Analyzer, the efficiency peaks at approximately 84.7% near full load in two-phase drive. Following your recommendation, we have also decided to include this efficiency evaluation directly in the revised manuscript. We present the newly rescaled figures here, featuring a zoomed-in curve from 40 W to 250 W input power:

(a)                                                                          (b)

System-level efficiency curve for the operation of the two-phase interleaved flyback rectifier with the proposed magnetic component: (a) efficiency curve across the entire load range, (b) efficiency curve from 40 W to 250 W input power.

Regarding the concern that this efficiency might indicate a fundamental flaw in a "coupled" transformer design, we would like to gently clarify the specific nature of our topology and the primary scope of this research.

First, the proposed magnetic structure is explicitly designed to be decoupled (non-coupled). By physically separating the flux paths within a single standard EE-core, the design intentionally avoids the circulating currents across multi-phase operation that can often limit traditional coupled designs.

Second, we would like to assure the reviewer that the ~85% efficiency is not indicative of a topological flaw, but rather a direct reflection of the specific hardware components utilized for this proof-of-concept prototype. The primary objective of this paper is to validate the feasibility of achieving independent, two-phase operation through structural magnetic decoupling within an integrated component. To ensure robust operation and protect the switches against surge voltages during these fundamental structural validations, we utilized standard, dissipative RCD snubber circuits.

Within the specific domain of flyback converters in this power class, an efficiency of approximately 85% is entirely consistent: [1] achieves a maximum efficiency of 83%, [2] an efficiency of 85.4%, [3] an efficiency of 85%, [4] an efficiency of 80-85%, [5] an efficiency of 88%, [6] shows an efficiency of around 80-85% using RCD snubbers, and [7] shows an efficiency of around 82%. Because our converter has not yet undergone comprehensive optimization, there is much possibility to improve the overall system-level efficiency in future iterations through targeted magnetic component optimization, the integration of active clamp circuits, or advanced control techniques.

Finally, regarding the recommendation to include an efficiency comparison with other state-of-the-art topologies, we highly appreciate the perspective of the reviewer. However, because experimental efficiencies in literature are heavily influenced by the specific selection of semiconductor devices, topology configurations, control methods, and auxiliary snubber optimizations, a direct numerical comparison of efficiency may not accurately reflect the isolated performance of the magnetic component itself. Instead, we have expanded our comparison to focus on the structural and operational trade-offs of state-of-the-art interleaved flyback topologies, which we have summarized in Table I of the revised manuscript. Please see the corresponding table as follows.

As recommended, we have added the efficiency curve (40 W to 250 W) and the corresponding analysis to Section V-B of the revised manuscript to provide insights on the system-level performance of the prototype.

| Before revision | After revision |
| --- | --- |
| (in Section I, 11th Paragraph) Structural decoupling approaches have been explored to bypass these circulating currents and accommodate the variable load requirements of USB PD, offering an alternative to control-based phase shedding. Some approaches design decouples phases by separating the flux paths using 3-D orthogonal flux core geometries [34], [35]. However, these custom cores constrain commercial manufacturability and increases fabrication costs. Another proposed solution is an integrated nonlinear coupled inductor designed to decouple four phases at light loads [18]. Achieving this variable coupling requires customized, multi-piece L- and T-shaped core assemblies with manually configured geometries, introducing tight assembly tolerances and manufacturing complexity. | (in Section I, 11th Paragraph) Structural decoupling approaches have been explored to bypass these circulating currents and accommodate the variable load requirements of USB PD, offering an alternative to control-based phase shedding. Some approaches design decouples phases by separating the flux paths using 3-D orthogonal flux core geometries [34], [35]. However, these custom cores constrain commercial manufacturability and increases fabrication costs. Another proposed solution is an integrated nonlinear coupled inductor designed to decouple four phases at light loads [18]. Achieving this variable coupling requires customized, multi-piece L- and T-shaped core assemblies with manually configured geometries, introducing tight assembly tolerances and manufacturing complexity. To clarify the operational trade-offs among these existing approaches, a comprehensive comparison of state-of-the-art interleaved flyback topologies and their magnetic configurations is summarized in Table I. |

| Before revision | After revision |
| --- | --- |
| N/A | (In Section I, at the bottom of Page 2) |

| Before revision | After revision |
| --- | --- |
| N/A | (after Section V-B) C. System-level Efficiency of the Proposed Interleaved Flyback Rectifier with the Integrated Transformer To evaluate the overall performance of the proposed topology, the system-level efficiency of the prototype converter was measured at a vin of 100 Vrms, VO of 48 V, and fs of 100 kHz up to an input power of 250 W using a Hioki PW8001 Power Analyzer. Fig. 19 illustrates the measured efficiency curve during two-phase interleaved operation. The converter achieves a peak efficiency of approximately 84.7% at full load. The primary objective of this study was to validate the fundamental magnetic decoupling of the integrated transformer, rather than optimizing system-level efficiency. To ensure robust operation during these initial magnetic validations, standard RCD snubbers were utilized. Losses associated with these snubbers are expected to reduce the measured efficiency. While the current prototype demonstrates volume reduction and independent magnetic operation between two phases, further efficiency improvements are anticipated in future iterations through the implementation of active clamp circuits, control techniques and optimization of transformer design. Fig. 19. Efficiency curve of the proposed interleaved flyback rectifier with the integrated transformer, up to full-load operation of 240 W. |

References mentioned for this comment:

[1] Y. G. Kim et al., "Low-Cost Universal Multi-Mode Digital Compensator Design for Flyback Converters With Full Digital Feedback," in IEEE Access, vol. 12, pp. 170840-170850, 2024, doi: 10.1109/ACCESS.2024.3498032

[2] Analog Devices, Inc., "AD-PS3803-RD — System Level Documentation," Analog Devices System Level Documentation. [Online]. Available: https://analogdevicesinc.github.io/system-level/solutions/reference-designs/ad-ps3803-rd/. [Accessed: Oct. 24, 2026].

[3] S. Y. Tseng and J. H. Fan, "Buck-Boost/Flyback Hybrid Converter for Solar Power System Applications," Electronics, vol. 10, no. 4, p. 414, 2021, doi: 10.3390/electronics10040414.

[4] S. F. Graziani, T. V. Cook and B. M. Grainger, "Isolated Flying Capacitor Multilevel Converters," in IEEE Open Journal of Power Electronics, vol. 3, pp. 197-208, 2022, doi: 10.1109/OJPEL.2022.3160049.

[5] Y. C. Li and C. L. Chen, "A Novel Primary-Side Regulation Scheme for Single-Stage High-Power-Factor AC–DC LED Driving Circuit," in IEEE Transactions on Industrial Electronics, vol. 60, no. 11, pp. 4978-4986, Nov. 2013, doi: 10.1109/TIE.2012.2224080.

[6] P. Thummala, H. Schneider, Z. Zhang, Z. Ouyang, A. Knott and M. A. E. Andersen, "Efficiency Optimization by Considering the High-Voltage Flyback Transformer Parasitics Using an Automatic Winding Layout Technique," in IEEE Transactions on Power Electronics, vol. 30, no. 10, pp. 5755-5768, Oct. 2015, doi: 10.1109/TPEL.2014.2379439.

[7] Y. C. Li and C. L. Chen, "A Novel Primary-Side Regulation Scheme for Single-Stage High-Power-Factor AC–DC LED Driving Circuit," in IEEE Transactions on Industrial Electronics, vol. 60, no. 11, pp. 4978-4986, Nov. 2013, doi: 10.1109/TIE.2012.2224080.

- The paper should present PFC voltage and current waveforms to substantiate the power factor claims. Typically, an uncontrolled rectifier with capacitive filtering yields a power factor of only 0.6–0.7 in the absence of PFC modulation.

We sincerely thank the reviewer for the valuable feedback. We completely agree that the power factor (PF) and input-current THD are critical metrics for PFC flyback rectifiers. Our prototype was constructed without a grid-side EMI filter to focus on validating the magnetic decoupling of the proposed integrated core, making it difficult to capture a perfectly clean, continuous sine-wave input current. However, the experimental waveforms in Fig. 18(a) clearly demonstrate the inherent PFC envelope tracking the input AC voltage. Because the inherent DCM PFC mechanism is already a well-established characteristic of this topology, maximizing the absolute value of the power factor was not the primary focus of our physical prototype. We would also like to emphasize that the proposed magnetic integration does not alter the fundamental electrical principle of the converter. The inherent DCM PFC mechanism remains identical to that of a conventional topology utilizing discrete transformers. To verify that the proposed integrated magnetic structure preserves this inherent PFC capability, we have conducted a comprehensive simulation in PLECS Standalone 5.0 with a standard differential-mode EMI filter (Lf=100 μH, Cf=1 μF) to obtain an accurate estimate.

As shown in the table below, we also compared the proposed integrated transformer against a conventional interleaved flyback baseline using two discrete transformers with matched magnetizing inductance Lm of 82.8 μH and turns ratio (n=3). The topology with the proposed magnetic component can preserve PFC, exhibiting PF and input-current THD performance that closely matches the conventional discrete setup across the extremes of the operating range, maintaining PF >0.95 at all representative points.

We have added PFC waveforms as well in various operating points for a visual reference for the reviewers. We have revised Section III completely to reflect these changes as indicated below.

(a)                                                                           (b)

Key PFC Waveforms of the proposed two-phase interleaved flyback rectifier showing vin, vo, iin, iS1 and iS2, and vds1 and vds2 with vin, Vout, and PO of: (a) 100 Vrms, 48 V, and 240 W and (b) 240 Vrms, 48 V, and 240 W.

| Before revision | After revision |
| --- | --- |
| N/A | (in Section III-B, after the 1st paragraph) Furthermore, to substantiate the PFC capability of the proposed integrated structure, a performance analysis was conducted across several operating points. For reference, the proposed design was compared against a conventional two-phase interleaved flyback rectifier utilizing two discrete transformers with an identical n = 3 and Lm=82.8 μH. To extract clean grid-side AC waveforms and accurately evaluate harmonic distortion, a standard differential-mode EMI filter withLf=100 μH, Cf=1 μF was incorporated into the simulation model. As summarized in Table IV, the proposed non-coupled integrated transformer maintains a high PF of >0.95 and low current THD across various input voltages and load conditions. Crucially, the fundamental decoupling of the magnetic structure ensures that its PF and THD performance remains nearly identical to that of the conventional discrete implementation and additionally validates the claims of inherent PFC operation in DCM in Section II. To provide a clear visual reference of this capability, the simulated PFC waveforms at the operating points of vin, VO, and PO of 100 Vrms, 48 V, and 240 W, and 100 Vrms, 48 V, and 240 W are shown in Fig. 13(a) and Fig. 13(b) respectively. (a) (b) Fig. 13. Key PFC Waveforms of the proposed two-phase interleaved flyback rectifier with vin, Vout, and PO of: (a) 100 Vrms, 48 V, and 240 W and (b) 240 Vrms, 48 V, and 240 W. |

- End of the response letter-